The authors express their sincere thanks and appreciation to the editor and the reviewers for providing their thought provoking suggestions and comments on the paper for improving its quality and acceptability. Our response to individual reviewers is detailed below.

|  | Editor's Comment | Authors' Reply |
| --- | --- | --- |
|  | The reviewers made several important comments and suggestions. The authors have to address each reviewer comments in detail. Also authors have to properly justify the proposed method with respect to existing literature on this topic. | All concerns/comments/suggestions of the reviewers have been addressed in the revised manuscript. Our response to individual points is detailed below. The importance and contributions of the proposed technique have been further highlighted in response to the reviewers questions regarding the same. |

Reviewers' Comments:

| I | Reviewer: 1 | Authors’ Reply |
| --- | --- | --- |
|  | 1. The last paragraph of section I should be revised to reflect the content of the following sections. | The numbering error has been corrected. |
|  | 2. The key point of this paper is the match of KEs from PMUs with offline calculation. But only stored and measured KEs are used, another indicator (such as fault location or rotor speed) should be included to gurantee the selected MOD is the right one. | Relaying on the fault location for MOD identification would be inefficient because a systematic ranking of the most-probably MODs is not always guaranteed by such a technique especially in the context of changing operating conditions. On the other hand, the MOD clearly depends on two things- a) energy distribution among generators at the instant of fault clearing, and b) potential energy absorbing capability of the system which depends on the post fault topology. Normalized kinetic energy (KE) captures the information regarding the first one. That's why it has been used for ranking of the most probable MODs. (∆VPEKEcorr) which has been used to identify the correct MOD, can capture the potential energy absorbing capability of the post fault system. Normalized kinetic energy has several other advantages- a) Normalized kinetic energy of generators remains almost same for different types of fault, and b) Normalized kinetic energy of generators remains almost same for small variation in fault duration. Therefore, it is sufficient to calculate normalized kinetic energy for one type of fault in offline and it can be compared with any type of fault in case of real faults in grid. In [28], we have proved these two advantages of KE theoretically and with simulations. |
|  | 3. An overall time cost evaluation should be included to show that the method can be used REAL TIME. | Authors thank the reviewer for the suggestion. It has been included in the revised paper in Section V. |
|  | 4. In table2, there are too many contingencies with too many generators in MODs. Generators, which tend to be out of step, are referred to as the MOD. It is unbelievable that all generators would be out of step. The results should be double checked. | In some fault cases, it has been seen that MOD contains a large number of generators. It may be noted here that just because a generator is part of an MOD, does not imply that the generator will become unstable. It may be the case that the generator is severely disturbed. Generally very few generators of the MOD group becomes out of step at first. In IEEE 39 bus test system, generator number 10 is the aggregated model of multiple generators (may be an external system). The inertia constant of this generator is 500 s, which is much higher as compared to other generators. Therefore, other generators with low inertia constant accelerate faster as compared to generator 10 and those generators frequently tend to be out of step with generator 10. Further, it has been observed that there is a weak inter area mode in this system, in which rest of the generators swing in opposite phase with the generator 10. And, it is very much well known fact that weak inter-area modes are prone to separation after disturbances. In most of the cases, generator 1 and 8 are also not included in the MOD as these two are strongly connected with the generator 10 which has very high inertia. |
|  | 5. Another case study should be used to show the possible application of this method in large scale power system. | The method has been tested for 57 machine 246 bus Northern Grid of India. It has been included in Section IV of the revised paper. |
|  | 6. Regarding the advantages in section V, almost all of them are with the TEF itself instead of the novelity of this paper. | The authors accept the reviewer comment. Therefore, that section (Section VII of the revised paper) has been modified to reflect the advantages of the proposed strategy, including the advantages of the original TEF technique. |
| II. | Reviewer: 2 |  |
|  | This paper presents an interesting approach for computing transient stability margin based on lookup table of Modes of Disturbances (MODs). The main advantage of the proposed concept according to the authors is that it overcomes difficulties associated with realtime calculations of Controlling Unstable Equilibrium Points (CUEPs). 1. Generally speaking, any advancement in the area of the realtime, TEF based, transient stability control, especially if it could use existing PMU measurements, is of great interest. Consequently, I encourage the authors to continue developing their ideas in this area. | Authors thank the reviewer. |
|  | 2. My concerns with this version of the paper would be primarily related to aspects of practical implementation, having in mind that the paper is oriented towards realtime control of transient stability. In large power systems (for sake of example let’s consider the U.S Eastern Interconnection), with thousands of generators, would accuracy and applicability of the proposed method be challenged by a) lack of PMUs at majority of generator’s locations | The reviewer has raised an important concern, which cannot be addressed in the current scope of the paper. However, our comments on this are as follows: As the TEF method needs the information on kinetic energy of the generators, it needs measurements from the generator buses. Generators located distant from a particular disturbance, exhibit relatively simple dynamics that can be approximated using aggregated models. The parameters of the aggregated model may have to be updated regularly, using SCADA measurements. On the basis of a smaller number of strategically located measurements from within the reduced generators, the energy (speed) of the aggregated generator can be approximated. The knowledge of the internal EMF (regularly updated using SCADA measurements) and its angle (approximated by a weighted average of the available PMU measurements) can further enable the calculation of the potential energy margin. Generators located adjacent to the disturbance would show complicated dynamics, and therefore PMU measurements would be required for such buses. |
|  | b) computational complexity of calculating MODs and CUEPs for faults at each location of the grid | Offline data base for large power systems would be constructed after appropriate contingency filtering strategies. For stable cases, the MOD would not need as regular updating as unstable/severely disturbed contingencies. In the online stage, the MOD would depend on the fault location and post-fault topology. So this could be used to reduce the search space within a large dataset. The procedure of MOD identification has been given in detail in Sec II-C of the revised paper. |
|  | c) estimation of postfault topology based on limited number of PMUs at large systems | Authors assumed that the entire network (high voltage buses) would be observable through PMUs in near future. Optimal PMU placement requires around one-third of the number of buses to be observed. As per the knowledge of the authors, utilities in India and China plan to locate PMUs so as to make all high voltage and generator buses observable through PMUs. In this context, the authors have earlier proposed a technique to identify the tripped line following a fault, quickly with PMUs located only at generator buses ([28] of the revised paper). The advantage of the method is that even if the method fails to identify the tripped line correctly, the wrongly identified line shows similar dynamic behavior to the actual tripped line. However, adequate load modeling must be in place for such a scheme to succeed. |
|  | d) varying unit commits/generation dispatch | Any online dynamic security/transient stability assessment strategy is valid only for that generator combination for which it was designed. Depending on the unit commitment, some generators may be turned on and some may be shut down. Therefore, prediction and control of transient stability methods may not work with past data. The main motivation of this work is to limit the importance of the offline computation to bare minimum and to take all decisions online. In this work, all calculations (transient stability prediction and control) are done online except the MOD identification for different fault locations which relies on a database created in the past. Even if the generator combination changes due to unit commitment, this step can be a success if the correct MOD is listed within the top few MODs ranked. Therefore, the challenge is to check whether the MOD for a particular fault location changes due to some changes in generator commitment. The identified MOD for a particular fault location but with changing scenario may be validated using various transient stability assessment techniques designed for multi-machine systems such as the Extended Equal Area Criterion [A]. [A] Y. Xue, T. Van-Custem, M Ribbens-Pavella, “Extended equal area criterion justifications, generalizations, applications,” IEEE Trans. Power Sys, vol.4, No. 1, Feb 1989. |
|  | e) variations of system loading conditions not only along daily loading curve but also due to seasonal loading variations. | Authors agree with the reviewer. As the method uses the MODs identified offline, therefore the method will work only if the MODs identified offline remain same. For a particular fault location, the MOD depends on several parameters- system topology, loading conditions, generators connected to the grid and their prefault output. These parameters undergo change due to daily, seasonal load variations, line status (system topology), and unit commitment. Therefore, the MOD is to be identified for different fault locations in regular interval periodically. |
|  | 3. The concept of “maximum sensitivity of the energy margin to generation shedding in different plants” (Page 4, Section IIIc) should be better explained. | The concept of maximum sensitivity of the energy margin to generation shedding has been taken from [19] and [24] of the revised paper. The same is re-iterated below. Energy margin ∆V is not directly dependent on shedding on kth generator Pmk. However, it depends on variables Meq (equivalent moment of inertia), Pik (=Pmk-Ek2Ykk), θi (angles of ith generator at the CUEP). These variables depend on shedding on kth generator. d(∆V)dPmk=0.5ωeqshed2dMeqdPmk-θku-θkcldPikdPmk+dθiudPmk(i=1n-Pi+j=i+1n{Cijsinθiju-Dij[(sinθiju-sinθijcl)θiju-θijcl-θi+ju+θi+jcl(θiju-θijcl)2+θi+ju-θi+jcl(θiju-θijcl)cosθiju]}) dMeqdPmk can be found if reduction in inertia of kth generator is assumed proportional to shedding in kth generator. dPikdPmk can be approximately taken as 1. dθiudPmk can be approximated as (i,k) element of (J-1), where J is the Jacobean with only active power equations. Inverse of J is already calculated during calculation Stable Equilibrium Point and the CUEP. This sensitivity is to be calculated for all the generators of the MOD. The generator which provides maximum sensitivity of ∆V, should be chosen for shedding. This calculation takes around 0.0003 s for each generator. If the number of generators in the chosen MOD is high, then it may take time. Therefore, the generator is to be chosen offline for each MOD and it will be selected for shedding online. |
|  | Is this calculation being performed in a centralized manner or distributed among several regional control centers? | All the calculations are done in a centralized manner. |
|  | How fast this sensitivity calculation needs to be (with recalculation of (Vcr -V(t)), in order to allow time for feedback control signal to be sent to trip selected generator(s)? | The reviewer is directed to Table 12 of the revised paper. The maximum allowable delay including the data transmission and computation burden depends on various factors such as the MOD, fault location etc. In some cases, it has been seen that despite significant delays the control action is able to stabilize the system, while in other cases the delay is crucial in determining the beneficial impact of the control actions. |
|  | Did simulations detect any cases in which the proposed approach underestimated the generator shedding requirement (please expand your thoughts related to Page 6, Step 8, second column, paragraph 1)? | Yes. 3 out of 8 cases under estimated the generation shedding requirement. In those cases, if the exact amount obtained from TEF is shedded, the control action would not be able to stabilize the system. Suppose there are 5 units of 200 MW each in a plant. a) The correct amount of shedding required (obtained from time domain simulation) is 190 MW but the proposed method predicts 150 MW. Solution: Trip one unit, system stable. b) Correct amount of shedding required is 190 MW but the proposed method predicts 210 MW. Solution: Trip two units, system stable, but tripping of one unit unnecessary. c) Correct amount of shedding required is 210 MW but the proposed method predicts 180 MW. Solution: Trip one unit, system unstable. d) The correct amount of shedding required is 210 MW but the proposed method predicts 240 MW. Solution: Trip two units, system stable. Thus, the primary implications of the error between proposed generation shedding prediction method and the actual shedding required, are the unnecessary tripping of an extra unit or ineffective control action. Further research is required to confront these challenges. |
| III. | Reviewer: 3 |  |
|  | The paper proposes a TEF based method to predict transient stability and mitigate the situation if the system is found to be vulnerable. At the beginning of the paper it is not clear whether the prediction is made before the event happens or after the event happens. This should be clearly stated at the beginning of the paper. | The prediction is made after a fault occurs. It has been included in the penultimate paragraph of Sec I of the revised paper. |
|  | In the second column of the first page under the point number (3), I believe that the authors mean "quantitative" not "qualitative" please check this. | The authors thank the reviewer for pointing out the error. It has been corrected. |
|  | If the power system is in a secure state, it does not go unstable for a single contingency. The scenario addressed in the paper is that a single contingency occur and the proposed method is used to determine the transient stability. This means that the power system was not in a secure state when the contingency occurred. The method proposed is for corrective control not preventive control. Please clarify these points. | We agree with the reviewer. The power system may not always be in a dynamically secure state, and the techniques proposed in the paper are meant to be applied for corrective action after a fault in the event of impending systemwide instability. |
|  | The proposed method involves a heavy computational burden given the condition that the fault has already happened and a quick decision needs to be taken to make corrective actions. Authors have not addressed this critical issue. | The major computational burden arises in the conventional TEF method from the calculation of the controlling unstable equilibrium point, which involves integration of the faulted and post fault system. However, this has been eliminated in the online portion of the proposed method and therefore the computational burden reduces significantly. A detailed analysis of the computational time has been included in Sec V of the revised paper. |
|  | The KE of the system after 0.1s is calculated in the proposed method. The rational of this choice is not clear. Does it depend on the system? There are standard PMU reporting rates. How do the authors suggest to obtain the data at exactly 0.1s instant. | Generally fault is cleared in 0.08-0.1 s (1 cycle for relay operation, 3-4 cycle for breaker operation in a 50 Hz system). Stored KE is calculated using speed data of 0.1s. For a 60 Hz system, the fault duration may be taken as 5cycle (5/60 s) for both stored and measured KE. If the PMU sampling frequency is at least 50 Hz for a 50 Hz system and 60 Hz for a 60 Hz system, then 5cycle data (0.1s for 50Hz system) is required. However, measured data may not be of 0.1s exactly as there may be error in detecting the exact fault inception point. The benefit of using normalized kinetic energy is that apart from its other advantages as mentioned in [28], it remains unchanged due to small variations in the fault duration, even if the actual kinetic energy varies. |
|  | What is the effect of data transmission delays on the proposed method? | The reviewer has raised a pertinent point. Due to lack of space, it has not been included in the revised paper. Kinetic energy of the generators from which the shedding is to be done, reduces with time. But if the delay increases, amount of generation required to be shed generally increases. It's because of less time available to decelerate and also the generator may not have much kinetic energy to influence the dynamics of the generators corresponding to MOD after its shedding. It is difficult to draw a firm generalized relationship between the required generation shedding and the communication delay. A rudimentary analysis of the amount of generation shedding required for different time delays (computation time=50ms, control action=50ms kept constant) yields the following results, for a particular fault. Fault: Location-Bus 2, CCT=0.319s, fault duration=0.33s Table 1: Impact of Communication Delay on Shedding Requirement Communication Delay (ms); Shedding Required (%). 30; 23. 40; 24. 50; 26. 60; 27. 70; 29. 80; 33. 90; 37. 100; 43. |
|  | Authors claim that the other available methods have the disadvantage that the trained algorithms are dependent on the system topology, but the proposed method does not. However, the MOD is determined by using a nearest neighbor search technique. The data for this search are generated offline and saved for real time use. Therefore the authors' claim is not valid. | In earlier works on transient stability prediction and control using artificial intelligence, all the decisions (stability classification and regression for severity) depends on machine learning and pattern recognition techniques. In this work, only the most probable MODs are ranked using the nearest neighborhood search. The actual correct MOD is identified using (∆VPEKEcorr) from the ranked MODs. Therefore correct MOD identification is not dependent on artificial intelligence, although the ranking does. If artificial intelligence (neighborhood search) performs worse than expected, ranking may not be efficient and the correct MOD may rank somewhat later than expected. But it will not be ranked towards the end of the list definitely. If the ranking of MODs are inefficient, then we need to check more MODs from the ranked ones using (∆VPEKEcorr). It will increase the computational burden. But final prediction of instability does not depend on pattern recognition. |
|  | The error of angle estimation is justified by citing a single reference where an error of 10 degrees is used as the acceptable value. This value is not a standard or well accepted value. The authors need to justify this choice for the particular application. | An analytical relationship between the error in estimation of angle and the potential energy calculated for multi-machine systems is difficult to obtain because potential energy depends on the difference between angles of generators rather than individual angle. Therefore, the sensitivity of energy margin to the error in angle/speed of generators was tested and provided in Sec VI of the revised paper. |
|  | Please discuss how the proposed method is superior to the methods in references [19,20,24,25,26,27,29,and 30]. There are also methods that use the samples of voltage magnitude after fault clearance to predict transient stability. What is the advantage of the proposed method over that approach. | Reference numbers have changed. Comparison has been given below: A. Fast Generation Shedding Equipment [15] a) it was proposed only for pumped storage or any other plants situated at a distant place. b)instability was predicted by observing the angle difference between pumped storage generator and a reference generator. The reference generator itself may go out of step for nearer faults. c)data used had sampling frequency of 600 Hz which restricts its application with PMU data.; a)applicable for any generator irrespective of their location b) prediction of stability depends on the total energy of the system after fault clearing and the maximum potential energy the system can absorb. As it is calculated online, there are less chances of error. c) prediction of stability does not depend on sampling frequency as it just need data of one instant after fault clearing. However, if instability is detected, angles and speeds need to be predicted at the instant of shedding. The accuracy of prediction will depend on the sampling frequency. In this paper, 100 Hz was used as recommended for 50-Hz system (120Hz for 60 Hz system) as per the IEEE Standard for Synchrophasor Measurements for Power System C37.118.1-2011.. B. Calculation of Generation Shedding Requirement of the B C Hydro System [16] a) it was intended for offline study purpose. For few fault locations, generation shedding requirement was calculated offline and stored in a look up table. However generation shedding requirement depends also on fault duration, fault resistance, fault type etc.; a) A detailed online calculation strategy is presented, which not only includes the stability prediction but also the amount of control action by taking into account the effect of fault type, duration, resistance, location etc.. C. A Hybrid Method For Generator Tripping [17] a) Uses look up table, detailed calculations were done offline. b) Stability was predicted by checking the angle difference with the biggest generator if it exceeds 1800. The angle difference with the biggest generator may vary depending on the distance between the two. If the biggest machine itself is going to be unstable, its difficult to predict stability.; a) A detailed online calculation strategy is presented, which not only includes the stability prediction but also the amount of control action by taking into account the effect of fault type, duration, resistance, location etc. b) prediction of stability depends on the total energy of the system after fault clearing and the maximum potential energy the system can absorb. As it is calculated online, there are less chances of error.. D. Fast Determination of Generation Shedding in Transient Emergency State [19] a) Sensitivity of energy margin in partial energy function with respect to generation shedding was used to find out generation shedding requirement. It was intended for use in offline to build up look up table. As the detailed calculations are done offline, it's quite difficult to include all the parameters that can influence severity. b) the method identify the critical machines as the machines having angles above the highest gap at the point of dynamic liberation point (maximum potential energy). It has been seen that when too many generators tend to become out of step, there is no very big gap and it may give wrong result as pointed out in [31]; a) A detailed online calculation strategy is presented, which not only includes the stability prediction but also the amount of control action by taking into account the effect of fault type, duration, resistance, location etc. b) the system was simulated for a fault duration slightly higher than the CCT. At the point of maximum potential energy, machines are ranked in descending order of anges. One after another machines from the top are added to the critical machine group and for each group CCT was found. The minimum error between CCT obtained from time domain simulation and the transient energy function was used to choose the group of critical machines defined as MOD in this paper.. D. Hydro-Qubec's Defence Plan Against Extreme Contingencies [18] a)This is not exactly related to transient stability prediction controls. Its more related to special protection schemes/remedial action schemes. Generation shedding, load shedding, reactor switching etc were proposed. Generation shedding was prescribed if the nearby lines are tripped for any reason. Number of generators to be tripped depend on the number of lines tripped. All the calculations were done in offline. hence control action doesn't depend on fault severity like fault duration, fault resistance, fault type etc and operating condition.; A detailed online calculation strategy is presented, which not only includes the stability prediction but also the amount of control action by taking into account the effect of fault type, duration, resistance, location etc.. E. Neural Net Based Determination of Generation Shedding Requirement [20] Disadvantage of the neural net based techniques has been given in Introduction section, first page, second column.; See Sec VII of the revised paper for a comparison with AI based techniques.. Samples of voltage magnitude after fault clearance- There are some methods which use voltage or power output after fault clearing to predict stability. These indices are indicative of fault severity and post fault topology. These are used in machine learning based techniques [3] as predictors. Problems associated with the machine learning based techniques are mainly due to computational constraints as faults are to tested for different fault location, fault duration, resistance etc. And also if the operating condition changes, the performance will deteriorate. Even if the stability status is predicted correctly, it will be difficult to predict the amount control action required. Whereas the proposed method performs detailed stability calculations online and hence effects of all the aforesaid parameters are taken into account. It can also provide information about fault severity so that efficient control action can be taken. |
| IV. | Reviewer: 4 |  |
|  | The paper proposes a method to implement real time stability control using Transient Energy Function method, using PMU data. It is proposed to use a lookup table created using offline simulations to overcome some of the problems associated with Energy Function method, such as the need for knowing the fault and post fault system configuration and out step generator groups. The need for simulation of post fault system is avoided by predicting the future rotor angles using a curve fitting approach. |  |
|  | Although the method is having some theoretical footing, in this reviewer’s opinion, the method is quite complicated for real time implementation and prone to errors at many points, as somewhat evident from the results. | The main and the difficult part of TEF i.e. computation of the controlling unstable equilibrium point has been eliminated in this proposed method. Therefore, the authors feel the method is simpler compared to BCU and other methods. Accuracy of the TEF method depends on correct computation of the CUEP and accurate modeling of power system. Though the method aims to simplify the computation method of the CUEP, but the final CUEP is same for both the proposed method and the BCU method. Therefore, the accuracy of the proposed method is almost same with the original TEF method, with inaccuracy arising out of errors in predicting generator angles at the time of shedding. This may impact the estimation of generation shedding which depends on the generator angles and speeds at the time of shedding. But the error in prediction of generator angles and speeds will have no impact on prediction accuracy of instability as the prediction is done using the measured data after the fault. |
|  | How would this be much different from machine learning type approaches? The creation of offline table requires consideration of different operating points and different faults, and selection of MOD still requires some classification type approach. Some of the machine learning type approaches has also shown that they can tolerate topology changes up to certain extent. | In earlier works on transient stability prediction and control using artificial intelligence, all the decisions (classification and regression) rely on machine learning and pattern recognition. In this work, only the most probable MODs are ranked using neighborhood search. The actual correct MOD is identified using (∆VPEKEcorr) from the ranked MODs. Therefore correct MOD identification is not dependent on artificial intelligence, but the ranking does. If artificial intelligence (neighborhood search) performs worse than expected, ranking may not be efficient and the correct MOD may rank somewhat later than expected. But it will not be ranked towards the end of the list definitely. If the ranking of MODs are efficient, then we need to check more MODs from the ranked ones using (∆VPEKEcorr). It will increase the computational burden. But final prediction of instability does not depend on transient instability. |
|  | Step3 in Section IIIB requires calculation of post fault stable equilibrium point for several probable cases, and it is proposed to use the procedure in Step1 in Section IIB, which is said to be a load flow like minimization problem. How would you apply this for a large system in real time? | Generally, the reduced admittance model is used in TEF which eliminates the non-generator buses and reduces the size of the system drastically. Prefault voltage and angles are used as initial values which are generally very close to the post fault solution. One iteration with Newton-Raphson power flow is expected to be enough. The internal emf are taken constant and hence only angles are taken as variable that further reduces the size. A detailed analysis of the computation time has been included in Sec V of the revised paper. |
|  | Generator speed is not usually provided as a standard quantity in synchrophasor data. You need to physically measure the generator speed and include that as an analog data. Or else, the generator bus frequency (which is provided by PMUs) is used as an approximation. How critical is the generator speed for the accuracy of the algorithm? | The reviewer has raised a pertinent point. Generally generator speed is measured at the plant site. It can be incorporated in the PMU data as PMU data based transient stability prediction methods need generator speed which is a important indicator of disturbance severity. If generator speeds are not provided in PMU data, it can be calculated using dynamic state estimators. The sensitivity of energy margin with respect to errors in the generator speed/angle has been analysed in Sec VI of the revised paper. |
|  | Even the generator rotor angles are not directly measured, they need to be estimated from the terminal measurements and machine impedance parameters. Can you use the generator terminal voltage angles instead of rotor angles? | Terminal voltage angle can't be used as the potential energy depends on the internal angle and voltage rather than the terminal voltage angle. |
|  | The presentation quality of the paper has suffered from the effort to keep it short. Lack of background make this paper difficult to understand for a person who is not very familiar with energy function methods. The theory presented in Section IIA is neither explained nor justified. Also no references to related work are provided. This section is very abrupt. The same comments as above are applicable to Section IIB. | Sec II has been modified in the revised paper. As the MOD and BCU methods are taken from the established literature in this area, they have been briefly described. |
|  | Define Symbols before using them or immediately after (for ex. second last para in the introduction, what are Delta_Vpe and KEcorr?). Acronyms must be defined at the first use (for ex. CUEP in the abstract, BCU in introduction, etc.). As many abbreviations are used, it may be good to have a list of symbols and abbreviations at the beginning. | The authors thank the reviewer for the suggestion. Due to space constraints, it is not feasible to include a separate Nomenclature section at the start of the paper. However, descriptions have been given for each variables and abbreviations wherever they are used for the first time. |
|  | In equations (1) – (4) use of indices i and j are confusing. First it is mentioned that machines i and j are the unstable machines, but then i is used as an index in (2) and (4). What is the minimizing function f_i^PF(teta)? Many of the variables in (5) and (6) are not defined. What are Cij and Dij? What is E? | The error in the index has been rectified in the revised paper. fiPFθ=Mid2θidt2=Pi-j=1,≠in(Cijsinθij+Dijcosθij)-MiMTPcoi This description of fiPFθ has been given in eq. 7. The equation has been referred in the II-A (sec II-A, second last para) where it has been used. All the variables have been defined after eqn. (1) in the revised paper. |
|  | The transient energy function in (9) is said to be well established, but no reference is given. | Reference for the energy function has been included in the revised paper. |
|  | In the conclusion, it is claimed that the main contribution of the paper is “calculation of CUEP in real time”. As I understand the MOD method of calculating CUEP presented in Section IIA is not new. | MOD method was developed around three decades ago. It uses the angles and the deceleration of the generators after fault clearing to predict which generators are going to be unstable. The set of generators which tend to be unstable are called the MOD. The method is heuristic in nature and very often it gives wrong MOD. Then Pavella et. al proposed a robust method to identify the critical clusters of generators (i.e. MOD). But this method also need to test many combinations of generators and for each combination extended equal area criterion was used to find the critical clearing time. This method also suffers from huge computational burden, and can't be applied in real time [31]. In this paper, during the offline stage the MOD is identified for each fault location in an unified manner as presented in Sec II-C. During the online stage, following the occurrence of a fault, the probable MODs are ranked using the normalized KE and the correct MOD identified from among them using (∆VPEKEcorr). This eliminates the need for integration of the faulted and post-fault system, as required in the BCU method. Thus, the computational burden is reduced significantly, which enables possible application of the TEF method in a real-time context. |
|  | When you shed generators to avoid instability, how would you keep the load generation balance in a practical system? You probably need some load shedding subsequent to the generator shedding. (In 39 bus test system, this is not visible as the lost generation usually supplied by the large equivalent generator) | It is assumed that system operators keep enough primary frequency reserve to meet the loss of the largest generator of the system or some percentage of the total generation. After shedding of some generation as mandated by the proposed strategy, primary frequency control is supposed to balance the mismatch between the generation and the load. Alternative, in the absence of primary frequency response, under-frequency based load shedding may be carried out. |
|  | In the introduction “The unstable equilibrium points of the generators corresponding to the MOD and the stable equilibrium points of other generators form the Controlling Unstable Equilibrium Point (CUEP).“ ..???? Is something missing here? | The reviewer is directed to Sec II-A and specifically eqn (3). We re-iterate it as follows: If machines p and q (in an m-machine system) tend to be out of step after fault clearing, then Mode of Disturbance (MOD) is generator group {p,q}. The vector of rotor angles [θ1s θ2s θ3s…θps…θqs.. θms] represents the post fault stable equilibrium point (SEP) which may be computed from the post-fault steady state conditions. Unstable equilibrium points of the pth and qth generator are π-θps and π-θqs. So, stable equilibrium point of other generators and unstable equilibrium point of generators corresponding to MOD (i.e. pth and qth generator) form the approximate CUEP. θUMOD=[θ1s θ2s θ3s …π-θps…π-θqs… θms]; (1). However, this CUEP is not in Centre of Inertia reference frame and the method described in Sec-IIA can be used to get the exact CUEP. |
| V. | Reviewer: 5 |  |
|  | This paper presents a real time control method for transient stability using PMU. I understand that the proposed method just follows the basic theory of the transient energy function method but it is difficult to find special new findings in the paper. | MOD method was developed around three decades ago. It uses the angles and the deceleration of the generators after fault clearing to predict which generators are going to be unstable. The set of generators which tend to be unstable are called the MOD. The method is heuristic in nature and very often it gives wrong MOD. Then Pavella et. al proposed a robust method to identify the critical clusters of generators (i.e. MOD). But this method also need to test many combinations of generators and for each combination extended equal area criterion was used to find the critical clearing time. This method also suffers from huge computational burden, and can't be applied in real time [31]. In this paper, during the offline stage the MOD is identified for each fault location in an unified manner as presented in Sec II-C. During the online stage, following the occurrence of a fault, the probable MODs are ranked using the normalized KE and the correct MOD identified from among them using (∆VPEKEcorr). This eliminates the need for integration of the faulted and post-fault system, as required in the BCU method. Thus, the computational burden is reduced significantly, which enables possible application of the TEF method in a real-time context. This is the principle contribution of the paper. |
|  | 1. This reviewer think that the method is based on the well known transient energy function method but the authors have not properly reviewed the past studies. This reviewer think that [A1] below should be cited. The method works in real power system for 20 years. [A1] H. Ota, et al., “Development of transient stability control system (TSC system) based on online stability calculation,” IEEE Trans on Power System, Vol. PWRS11, No. 3, pp. 14631472, 1996. | The authors thank the reviewer for the suggestion. The suggested reference has been included as reference number [23] in the revised manuscript. |
|  | 2. The authors use index (dVpe/KEcorr) to identify MOD in step 3 in section III. B a). The index appears several times in the paper before this part but is not clearly identified; the explanation is finally given after step 5. This reviewer think that this part is not original but important for this paper, and should be more explained using equations. In general, the numerator (dVpe) and denominator (KEcorr) show different performance, and it is questionable if the index shows an expected performance. I think that KEcorr may work better than dVpe, but both of them may contain unacceptable errors depending on power system conditions. The authors should explain how the identification of MOD is correctly carried out from the point of view of the individual factors using more analysis data. | ∆VPEKEcorr has been defined in Sec-III where it has been used first. At time t, ∆VPEt=VPEcr-VPEt=maximum potential energy the system can gain further and KEcorr(t) is the corrected kinetic energy calculated with speeds of that instant. VPEcr is the maximum potential energy calculated with generator angles at the CUEP. VPEt is the potential energy at time t. The effect of fault duration and fault resistance is taken into account if the corrected kinetic energy KEcorr is used. However transient stability after a disturbance also depend on the post fault topology (potential energy absorbing capacity), effect of that is not considered in KEcorr. Sometimes, it has been seen that a generator may gain high kinetic energy during fault but it does not lose synchronism due to strong connection with other generators. If KEcorr is considered for choosing the correct MOD, then that generator may be wrongly included in the list of generators comprising the MOD for that fault location. It has been seen in our study also. Generator 1 does not lose synchronism for a fault at that generator bus (bus 2), but other generators (3,4,5,6,7,8,9) lose synchronism. It's because of the strong connection with the generator 10, which does not accelerate much due to its high inertia. While KEcorr indicates the kinetic energy after the fault for separation, ∆VPE indicates the amount of potential energy it can gain at maximum (the maximum amount of potential energy the system can absorb). For a correct MOD, ∆VPE will be less and KEcorr will be high. Therefore, (∆VPEKEcorr) is taken for identifying the correct MOD that takes both fault severity and the potential energy absorbing capability into account. |
|  | 3. The explanation of (KEcorr) after step 5 above shows that the paper use the concept of the two machine equivalent. If so, the authors should refer to the research study by M. Pavella, et al. I think that the proposed method is similar to this work as a result. | It has been included in the reference of the revised paper as reference number [13] The total system kinetic energy does not lead to system separation. Part of the energy is spent on acceleration of the total system inertia. To the authors knowledge, the corrected kinetic energy was first developed in [14]. However, the work described in [13] was also a significant development of the theory of transient energy function. |
|  | 4. I think that the most critical part of the proposed method is “Estimation of stability” in section III. B. b) and c). This part is theoretically acceptable but the computation models themselves are erroneous and require more reliability to be used in real system. The first factor is that the stability judgement by the energy function itself is erroneous, even when the accurate CUEP is obtained using reliable method. Therefore, its use is usually limited to screening purpose before the final stability judgement. | Authors agree with the reviewer that TEF is not as accurate as the time domain simulation method. However, time domain simulation methods are unlikely to be applied in real time in near future due to higher computational burden and less time availability to initiate control action. Even if time domain simulation is run in real time with very fast computer, it can only give the stability status without giving any stability margin or an estimation of generation shedding required. While artificial intelligence has the potential to provide the amount of shedding required, considering different fault locations, fault durations, fault resistances and post fault topology, the computational burden becomes very high due to the curse of dimensionality. The only additional error in this method is the error in prediction of generator angle and speed. The authors feel that the error in predicting angles and speed was found to be quite small and suitable for real time application. Another possible solution is to integrate the post fault system with parallel computation up to the instant of control action. If reduced admittance matrix is used for admittance load model, network size reduces to the generator buses only and it may be possible to integrate post fault system equations for ~0.2 s. If the computation is possible in a very short time, integration of post fault dynamic equations may be applied instead of predictions of angles using curve fitting. For 10 machine, 39 bus test system with admittance load model, it takes 0.005s to simulate for 0.2s in PSS/E Ver. 33. For all test cases, the amount of generation shedding required from TEF and time domain simulation both are given in Tables 8 and 11. It can be seen that the error between the two is not very high. Considering the fact that generation shedding is done by tripping a single or multiple units, the authors submit that an error in order of 10-15% of the rating of the generator (it's not 15% of the total generation) is acceptable. |
|  | 5. The control method based on the energy function may be effective in many cases in my experience but cannot be useful unless the accuracy is guaranteed. The authors should add information about the accuracy of the proposed method compared with the exact numerical simulations. For example, the readers must be interested in the accuracy of the total amount of generator or load shedding, which should be compared between the proposed method and the conventional numerical simulations. | In each test case, the amount of shedding required obtained from TEF and time domain simulation (TDS) has been given. For instance, the reviewer is directed to Tables 8 and 11 of the revised manuscript. |
|  | 6. PMU data is also known to be unreliable, and therefore, it may worsen the accuracy of the proposed control method. Although I basically interested in the transient stability control method, the authors should more focus on the accuracy issues. | The authors agree with the reviewer that PMU data may be un-reliable under some circumstances. In that case, the proposed technique cannot be successfully applied. The accuracy of the TEF method has been compared with the time domain simulation. In each case, the amount of shedding obtained from TEF and time domain simulation (obtained using repeated simulations) both have been given. The reviewer is directed to Tables 8 and 11 of the revised manuscript. |
|  | 7. All reference papers should be cited in the main text. | All the papers have been cited in the revised paper and some new papers have been included on transient stability control. |