The authors express their sincere thanks and appreciation to the editor and the reviewers for providing their thought provoking suggestions and comments on the paper for improving its quality and acceptability. Our response to individual reviewers is detailed below.

|  | Editor's Comment | Authors' Reply |
| --- | --- | --- |
|  | The authors should further clarify the reviewer’s comments and suggestions. | All concerns/comments/suggestions of the reviewers have been addressed in the revised manuscript. Our response to individual points is detailed below. |

Reviewers' Comments:

| I | Reviewer: 1 | Authors’ Reply |
| --- | --- | --- |
|  | 1. No further comments. | Authors thank the reviewer. |
| II. | Reviewer: 2 |  |
|  | No further comments. | Authors thank the reviewer. |
| III. | Reviewer: 3 |  |
|  | Although the authors have upgraded the paper to clarify the questions by the reviewers, the following points are still unclear. (1) In the paper, the authors use potential energy function without definitions, and also introduce different approximations of CUEPs such as CUEP of equation (3), CUEP by BCU method, etc. For example, in step 4 in section III.B, CUEP must be defined by using equation number. Is that given by equation (3) and (6)? In step 5, the authors must define the potential energy function. Is it the energy function for multi-machine system or for two machine equivalents? These issues must be clarified. | A) Any approximation in the CUEP? It may be clarified that CUEP used in the paper is the correct or the relevant UEP for a particular fault, no approximation has been used. As there are many difficulties with applying BCU in real time, the proposed method uses a simplified way with prior information about the group of generators or the MOD. Equation (3) was written to explain that if the MOD is known, the CUEP can be calculated from equation (3). However, this is not in COI reference frame and therefore the procedure given in Sec-IIB is followed to calculate the correct CUEP. Correct CUEP is calculated from equation (6). In Sec IIB and Sec III, step 4, it has been clarified in the revised paper. B) Definition of Energy Function and Potential Energy Function- Energy function and potential function has been described in the revised paper, as given below- "Energy function for power system transient stability studies is well established [5, 14] and used for contingency ranking and other transient stability studies. It describes the total system transient energy for the post disturbance system. Vωi,θi=VKE+VPE =0.5 Meqωeq2-{i=1mPiθi-θis+i=1m-1j=i+1m[Cijcosθij-cosθijs+Dijθi-θis+θj-θjsθij-θijs(sinθij-sinθijs)]} (1) where, ωi=speed of ith generator, Meq= moment of inertia of two machine equivalent, ωeq= speed of two machine equivalent, E=internal emf, Pi=Pmi-Ei2Gii, θs=post fault stable equilibrium point, m=number of generators, Cij=EiEjBij, Dij=EiEjGij. The energy function consists of four terms- a) 0.5 Meqωeq2: total corrected kinetic energy of the generators. b) i=1mPiθi-θis: total potential energy of the generators. c) i=1m-1j=i+1mCijcosθij-cosθijs: stored magnetic in all branches d) i=1m-1j=i+1mDijθi-θis+θj-θjsθij-θijs(sinθij-sinθijs): approximate dissipated energy of all branches. All these energy terms are deviation from steady state values and calculated in COI reference frame. the first term is the kinetic energy and depends on only generator speeds. Last three terms together form the potential energy of the system which depends only on generator angles." C) Energy Function for Multi-machine or Two machine equivalent? Energy function used in this paper is for multi-machine system. As the correct MOD is known, the kinetic energy responsible for system separation can be used instead of total kinetic energy. This kinetic energy is called "corrected kinetic energy". In two machine equivalent, equal area criterion is used (static extended equal area criterion or dynamic extended equal area criterion), in this paper it has not been used. |
|  | (2) It is known that CCT estimated using the energy function at CUEP of (3) sometimes includes large errors. If the authors use it, some comments should be added. | In general CCT estimates using TEF is within certain margin (below 5%, give vittal's paper's reference). However as the authors pointed out in [26], BCU method converge to a wrong CUEP in certain difficult situations like fault in highly power deficit area etc. Basically it indicates the BCU method predicts stable generators to be out of step (higher angles at the CUEP). However, BCU method is used here in offline to get the approximate CCT which is further validated by time domain simulation (II-C, Identification of MOD in Offline). As the correct MOD is identified in offline and MOD technique is valid for power deficit cases also, the method is supposed to give a good estimate of the CCT in the critical cases as well. Another reason of error is using total kinetic energy instead of the kinetic energy responsible for system separation. However as the kinetic energy of the two machine equivalent (machines of MOD forms the out-of-step group, rest of the generators form the stable generators) has been used in this paper, accuracy improves. One more reason behind slight difference between CCT obtained from TEF and TDS is that approximate modeling of the power system. The authors in [35], have shown that with simplified exciter model TEF can give a good estimate of the actual CCT of a power system with higher order generator model and fast acting exciter. Average error in total 6 cases in estimated CCT was 0.0092 second. However the authors accept that TEF (at least in its present form) can't be as accurate as time domain simulation. |
|  | (3) The authors rely on the following equation: (dVcl＝Vcr－Vcl) in section III.B.b with the statement of “If the energy of the system at fault clearing instant (Vcl) is less than the energy at the CUEP (Vcr), then dVcl is positive and the system will remain stable.” In this statement, “the energy at the CUEP (Vcr)” should be “the potential energy at the CUEP (VPEcr).” In this case, positive energy margin (dVcl＝VPEcr－Vcl >0) is the stability condition as the authors state in the paper. (4) The authors use the energy margin (dVcl＝VPEcr－Vcl >0) for evaluating CCT and for stabilizing control as well. The statement of “the energy margin (V(t)-Vcr)” should be (Vcr -V(t)) or (VPEcr – V(t))?. | The authors agree with the reviewer that critical energy or the energy at the CUEP is the potential energy at the CUEP as it is assumed that if the system reaches to the CUEP for a critical fault, kinetic energy becomes zero (as it is an equilibrium point). Therefore potential energy and total energy at the CUEP are equal. However, the authors have followed the past papers and books [24] where this has been described as critical energy, not the potential energy at the CUEP. The authors agree that VPEcr = Vcr, hence any of the two can be used. As kinetic energy at the CUEP is zero, (Vcr -V(t)) and (VPEcr – V(t)) are equal and any of the two can be used. |
|  | (5) There are no descriptions about how to evaluate V(t) above. In theory, V(t)=VPE(t)+KE. The authors must define the function forms of V(t). | Authors agree that description on TEF given in this paper, is very short. As the technique is well established and sufficient literature is available, relevant references have been given in the paper. Expression for V(ωi,θi) has been given in equation (1). Each variable has been defined after the equation. Significance of time is lost in energy function which is represented as a function of ωi andθi. ωi andθi are the values of speed and angles of ith generator at a certain instant t, therefore value of function V represents the transient energy of the system at that instant. Vωi,θi=VKE+VPE =0.5 Meqωeq2-{i=1mPiθi-θis+i=1m-1j=i+1m[Cijcosθij-cosθijs+Dijθi-θis+θj-θjsθij-θijs(sinθij-sinθijs)]} Detail description on calculation of energy function can be found in [24], Chapter 6 (Computational aspects of the transient energy function method for the classical generator model) and chapter 8 (Modeling improvements in the TEF method). Considering the length of this paper, the authors could not include all the computational details in this paper. |
| IV. | Reviewer: 4 |  |
|  | The responses provided to the reviewer's concerns should be included in the paper as well. | Authors thank the reviewer. As the length of the paper has already become 11 pages, authors regret for not being able to include some of the interesting and very useful discussions. |
| V. | Reviewer: 5 |  |
|  | The revised give much more information on the proposed energy function based method for transient stability prediction and control. The paper presents some interesting ideas, but I have the following comments: | Authors thank the reviewer for the appreciation. The comments given by the reviewer in the previous version regarding accuracy issues, inspired the authors to do detailed study on accuracy issues with TEF (with classical model, detailed model with exciter for different communication delay in WAMS based transient stability control) which will be covered in our next work which is in its completion stage. |
|  | Although authors claim the proposed method is do not suffer from the drawbacks of machine learning based approaches, I do not see a clear fundamental difference. In machine learning methods, information provided by large number of examples are retained in different ways, for ex. as weights of a trained ANN classifier, etc. In this approach example data is kept as a look up table. The solution is as good as the examples provided in both cases. | A. Generating offline data for faults of different duration, location, resistance, types consumes huge computation time. Therefore, offline data are generated for few operating conditions only. As CCT varies with operating condition and sometimes training and test data may not match, there is a possibility of error. There is no way to check whether the prediction is right or wrong. Like machine learning techniques, offline data for storing the probable MODs for each fault locations has been stored in look up tables and normalized kinetic energy gained by the generators act as a predictor in machine learning techniques. However, in this work offline data and normalized kinetic energy are used for getting most probable MODs. But the correct MOD is chosen using lowest normalized potential energy margin., As long as all the probable MODs are stored in the look up table, the method will give correct MOD and hence correct stability analysis. As the MOD depends mostly on fault location and less on operating condition, it is easy to generate the possible MODs for different fault locations. Unlike, machine learning methods, here offline data is used just for sorting the probable MODs, but correct MOD is not chosen using machine learning, rather normalized potential energy margin. B. As its impossible to generate offline data for every operating condition, data are generated normally for few operating condition, fault types, and duration. Therefore, most of the published literature are on prediction of stability status rather than fault severity (how much unstable). While in this paper, as current operating condition is used and stability is predicted after fault clearing, the impact of operating condition, fault duration, location, types, resistances etc. are inherently taken into account. ........................... Disadvantages- Authors agree that machine learning techniques also have certain advantages, especially in case of extremely big and complex system. With normal PC and small system size, computation time is small. If high performance supercomputers are used, energy function can be calculated for bigger systems in real time, but how much bigger size is matter of concern. In that case, there will be no option left except machine learning techniques. |
|  | It is not clear what type of generator models are used for simulation based validation. Have the higher order generator models been used for system simulations, although the stability assessment based of TEF assumes simplified models? This is critical information needed to assess the robustness of the method. | The reviewer has raised an interesting point. In this paper, the authors used classical models for both simulation and energy function calculation. It was clarified in the introduction section, "generator models used for simulation as well as energy function calculation, are classical models. However the method is also applicable to 2-axis model with exciter". The authors are presently working on impact of communication latency/time delay on effectiveness of WAMS based transient stability control, which is natural extension of this work. That work includes- accuracy of TEF with different latency, maximum allowable delay with which control action can stabilize the system. There we will include detailed models which will be communicated for publication shortly. Inclusion of high gain fast exciters needs inclusion of tuned PSS as well. Considering the length of the current version of this paper, the authors have decided to include the detailed model in the next work. However, MOD technique is also valid for 2-axis generator model with exciter. How to include detailed model with exciter in the proposed method with sufficient accuracy and manageable computation time, have been given below- A. TEF for detailed model with exciter- Energy function with detailed generator model with exciter has been given below- Vw,θ=0.5Meqωeq2-i=1nPmi-αiiGiiθi-θis+i=1n-1j=i+1nBijαij-cosθij+cosθijs-βijBijsinθij-sinθijs+i=1n-1j=i+1n[Gijαijθi+θj-θis-θjsθij-θijssinθij-sinθijs+ Gijβijθi+θj-θis-θjsθij-θijscosθij-cosθijs] Except α, β, G, B, rest of the terms are there in the energy function of the classical machine model, and defined in the paper. αij=Edi'Edj'+Eqi'Eqj', βij=Edi'Eqj'-Eqi'Edj', G, B are real and imaginary part of the reduced admittance matrix. Though Edi'and Eqi' varies with time after a fault, these are taken as constants. Magnitude of Edi'and Eqi' are taken as average of their values at fault clearing and the potential energy peak. Values of these emfs at the potential energy peak can be calculated in offline with simplified one-time constant equivalent exciter model or from time domain simulation (critically cleared) in offline. As Edi'and Eqi' are taken as constants and their values depend mostly on fault location (less dependent on operating condition) [35], it can be found offline. B. Exciter effect on Ed' and Eq' Due to exciter control action (and also armature reaction), Edi'and Eqi' change after fault clearing. If varying Edi'and Eqi' are used in energy function, it will need huge computational burden. Therefore, average value of Edi'and Eqi' at the fault clearing and the peak point is used in the energy function. Edi'=0.5(Edi.clearing'+Edi.peak') Eqi'=0.5(Eqi.clearing'+Eqi.peak') Values of the Edi'and Eqi' at the peak can be calculated easily with a simple one-time-constant equivalent exciter model as given in [35], or from critically cleared time domain simulation. As generators nearer to the fault experience the exciter effect prominently and the effect is less dependent on operating condition [35], it can be calculated offline for different fault locations (just like normalized kinetic energy stored in a look-up table in this method). C. Computational burden with detailed generator model and exciter Computational burden for energy functions for classical model has been given in the paper. However, the impact of exciter model and detailed generator model is an important issue. As explained earlier, total computational burden for detailed model will be higher as compared to the classical machine model. But, part of the calculations can be done offline, therefore computational burden for the online part remains almost same for detailed model. Lets define, X1=-cosθij+cosθijs X2=sinθij-sinθijs X3=θi+θj-θis-θjsθij-θijs Then energy function becomes- Vw,θ=0.5Meqωeq2-i=1nPmi-αiiGiiθi-θis+i=1n-1j=i+1nBijαij.X1-βijBijX2+i=1n-1j=i+1n[Gijαij.X3.X2+ GijβijX3-X1] Even for energy function with classical model, (X1, X2, X3) are calculated. As the calculations are simple arithmetic, computational burden depends mostly on the number of inner loops and the number of times (Number of generators in this case) it needs to be run. As the number of loops in the computations remains same and (X1, X2, X3) are also calculated for classical model, computational burden will remain almost same for detailed model. However, inclusion of detailed model with exciter increases computational burden in offline calculation part, as value of Edi'and Eqi' at the peak point is to be calculated offline. D. Impact of Exciter and Detailed Model on Accuracy of Energy Function- Use of classical model neglects the change in internal voltage due to exciter control action and also the effect of armature reaction. Therefore, TEF with classical model is more likely to produce pessimistic results for shorter fault duration and optimistic results for higher fault duration. Test Results With Detailed Machine Models (Without Exciter)- Accuracy of the methods is same with the original TEF method, as same energy function and same CUEP is used- The authors tested the accuracy of TEF with detailed model without exciter. Test results for two cases (one on inter area instability, another on plant mode instability) have been given below- Fault Location; Mode of separation; CCT with detailed model (time domain simulation) (s); CCT (TEF) (s). Classical Model; Detailed Model. 29; Plant mode; 0.11; 0.09; 0.1. 15; Inter area; 0.152; 0.166; 0.161. As the accuracy reduces for simplified TEF model ( but detailed generator model in simulation), the author suggests use of detailed model in energy function too. As the computational burden in online part is almost same with classical model, detailed model can be used in this method. Test Results With Detailed Machine Models (With Exciter)- Inclusion of fast acting exciters need inclusion of local PSSs as well. Authors have data of exciters in IEEE 39 bus test system, but don't have the PSS data. Proper tuning of PSSs will take time, hence authors didn't test the accuracy of the TEF with exciter. Accuracy of TEF with detailed generator model and exciters were tested in [35], summary of the results have been given below. Average values of Edi'and Eqi' (as explained above) were used in that paper. Number of Case Studies; Average Error between CCT (s) obtained from TEF and time domain simulation. 6; 0.0092 s. It can be seen that average values of Eq' and Ed' can be used with sufficient accuracy and manageable computational burden. As of now, we did not test it ourselves, but we are working on it to include in our next work. However, if the reviewer feel that it is necessary for completeness of this paper, we will include application of this method with detailed model in the final version of this paper. The authors have included future work in the revised version of the paper- "In this work, classical models have been used for both simulation and energy function calculation. However, as the MOD technique is valid for both classical and detailed model [24, 35], the method can be extended for detailed model also. Presence of exciter doesn't change the structure of transient energy function, changes only the magnitude of internal Edi'and Eqi'. Edi'and Eqi' of generators nearer to the fault affects transient stability prominently. Magnitude of Edi'and Eqi' are more dependent on fault location, lesser on operating condition. Therefore it can be calculated offline and stored in look up table and included in the energy function in this method." |
|  | Why it is necessary to replace single circuit lines in the 39 bus system with double circuit lines? What prevents the prediction of transient stability of the system with the original configuration? | In some cases, removal of the line in original test system causes islanding or post fault steady state instability. Single lines were replaced with double lines so that one circuit can be removed after a line fault. |
|  | No results are provided to verify the accuracy of online estimated CUEP, in comparison to that obtained with time domain simulation based approach (where accurate higher order generator models with exciters are used for the simulation). | The reviewer has asked an important question. This method doesn't change anything in the energy function, but simplify the process of calculating the CUEP with offline stored data, so that the method can be applied online. Therefore accuracy issues with this method are same as the accuracy with original TEF method with relevant CUEP. The basic assumption in the TEF method is that critically cleared post fault trajectory will pass through the CUEP (hence speed and kinetic energy at that point=0). This never happens in practical systems. But early investigations showed that the critically cleared post fault trajectory pass through nearby regions of the CUEP and speed of the generators (at least the generators with advanced angles) becomes close to zero. Therefore critical energy calculated at the CUEP becomes almost equal to the energy at the potential energy peak obtained from critically cleared post fault trajectory. As replied above, the authors are currently working on accuracy issues of TEF with classical and detailed model for different communication latencies. An important observation regarding this question, the CUEP used in TEF technique are closer with the CUEP obtained from simulation in normal cases (stability analysis without shedding). But when very high amount of shedding is done in case of plant mode instability, accuracy of the CUEP reduces. Critical energy deviates from the critical energy calculated with the CUEP obtained from simulation. All these details like accuracy of TEF, how to detect wrong results and how to calculate energy value in those critical cases will be covered in our next work which is in its completion stage. The CUEP calculated in this method is the same with the CUEP obtained from BCU. Accuracy of the CUEP in comparison with the CUEP obtained from simulation has been compared in [35]. Number of fault cases; Average error in generator angles at the peak point; Average error in CCT. 5; 23 degree; 0.0092 s. It may not be sufficiently accurate for transient stability control calculations in some critical cases. As per the authors' recent study, when very high amount of generation is shed in case of plant mode instability (only one or very few machines go out of step), in few cases TEF gave wrong result and correct result in rest cases. Those cases can be detected easily by the fact that stability degrades with increased shedding when TEF gives wrong result. It will be covered in detail in our next work. |
|  | The revised paper is very long; 11 pages is significantly longer than the normal length of 8 pages. |  |